SMART HELMET-BASED
MOTORCYCLE IGNITION SYSTEM

Product Requirements Document (PRD)

Version 2.0  |  Engineering Product Requirements

Prepared By: Yogesh  |  Date: August 2026

# Document Control

| Item | Value |
| --- | --- |
| Product | Smart Helmet-Based Motorcycle Ignition System |
| Document | Product Requirements Document |
| Version | 2.0 |
| Primary Goal | Prevent motorcycle ignition authorization when an authorized helmet is not worn |
| Target Detection Time | <500 ms |
| Target Wireless Delay | <300 ms |
| Target Communication Range | 2–5 m |
| Target Communication Success | 99% |
| Target Helmet Runtime | ≥30 hours |
| Primary Controller | ESP32 |
| Preferred Prototype Communication | ESP-NOW |
| Security | Unique device ID + pairing protection + encrypted communication |
| Status | Requirements baseline |

This PRD is a cleaned and structured version of the uploaded PRD. It preserves its scope, requirements, constraints, risks, success criteria, cost estimate and deliverables, while making the requirements easier to implement and verify. fileciteturn2file0L2-L16

# 1. Product Overview

The Smart Helmet-Based Motorcycle Ignition System is an embedded/IoT safety system designed so that a motorcycle can be started only when the rider is wearing an authorized smart helmet. Sensors inside the helmet detect helmet-wear status. The helmet controller sends authorization data wirelessly to the motorcycle controller. The motorcycle controller verifies the received information and enables the ignition relay only after successful validation. fileciteturn2file0L13-L22

# 2. Problem Statement

Many riders do not consistently wear helmets. A conventional motorcycle can normally be started without checking helmet use. This creates a safety gap. The proposed product enforces helmet use at the ignition-authorization stage by making helmet detection and authentication prerequisites for starting the motorcycle. fileciteturn2file0L23-L33

# 3. Product Objectives

| Priority | Objective | Success Direction |
| --- | --- | --- |
| Primary | Allow ignition only when an authorized helmet is properly worn | No authorization when helmet condition is invalid |
| Secondary | Prevent riding without helmet | Helmet condition is checked before start authorization |
| Secondary | Provide reliable wireless communication | Meet defined latency/range/reliability targets |
| Secondary | Keep helmet power consumption low | Target ≥30 h operation |
| Secondary | Keep installation simple | Modular helmet and bike units |
| Secondary | Maintain rider convenience | Fast detection and authentication |

These objectives follow the uploaded PRD. fileciteturn2file0L34-L42

# 4. Scope

| Included in Prototype | Excluded / Future Upgrade |
| --- | --- |
| Helmet-wear detection | Autonomous driving |
| Wireless communication | GPS tracking |
| Motorcycle ignition control | Accident detection |
| Rider/helmet authentication | Vehicle anti-theft |
| Helmet battery monitoring | Speed monitoring |
| LED status indication | Cloud/mobile advanced services |
| Buzzer alerts | Other future safety features |

The original PRD explicitly lists these inclusions and exclusions. fileciteturn2file0L43-L59

# 5. Target Users

Primary target users are motorcycle riders, delivery riders, college students, fleet operators and safety organizations. fileciteturn2file0L60-L65

# 6. User Needs

| User Need | Product Response |
| --- | --- |
| Rider should not be able to start without helmet | Ignition authorization depends on helmet status |
| Rider needs quick start after wearing helmet | Detection and wireless delay targets are defined |
| Rider needs clear feedback | LEDs and buzzer provide status/fault indication |
| Authorized helmet should be recognized | Unique device identity and authentication |
| Helmet should operate without frequent charging | ≥30 h battery target |
| System should tolerate normal riding conditions | Vibration, temperature and enclosure requirements |

# 7. Functional Requirements

## FR-01 — Helmet Detection

The system shall determine whether the helmet is properly worn.

| Item | Requirement |
| --- | --- |
| Primary sensing | Pressure sensor |
| Secondary sensing | Chin-strap sensor / magnetic switch |
| Possible alternatives | IR, capacitive sensor |
| Output states | WORN / NOT_WORN |
| Performance target | Detection completed within <500 ms |

The uploaded PRD lists pressure, IR, magnetic, chin-strap and capacitive sensing as possible methods. fileciteturn2file0L66-L79

## FR-02 — Wireless Communication

The helmet shall transmit authentication/status information to the motorcycle controller.

| Item | Requirement |
| --- | --- |
| Candidate technologies | BLE, NRF24L01, RF, ESP-NOW, ZigBee |
| Preferred prototype | ESP-NOW |
| Target range | 2–5 m |
| Target wireless delay | <300 ms |
| Reliability target | 99% communication success |

The source PRD identifies these candidate technologies and the 2–5 m range; its recommended stack prefers ESP-NOW for ESP32-to-ESP32 communication. fileciteturn2file0L80-L89 fileciteturn2file0L329-L340

## FR-03 — Authentication

The motorcycle shall verify that the received helmet is the correct authorized device and that the communication is valid.

| Check | Required Behavior |
| --- | --- |
| Correct helmet/device | Accept only configured authorized helmet |
| Authorization | Reject unknown/unpaired helmet |
| Communication validity | Reject invalid/corrupted communication |
| Failure | Engine/ignition authorization remains OFF |

These authentication requirements are directly stated in the source PRD. fileciteturn2file0L90-L96

## FR-04 — Ignition Control

| Condition | Required Output |
| --- | --- |
| Helmet worn + authentication valid | Ignition relay may be enabled |
| Helmet not worn | Relay remains OFF |
| Authentication failed | Relay remains OFF |
| Communication unavailable | Relay remains OFF |
| Critical system fault | Relay remains OFF |

The original requirement defines relay enable when the helmet is worn and relay OFF otherwise. fileciteturn2file0L97-L102

## FR-05 — Status Indication

| Indicator | Meaning |
| --- | --- |
| Green LED | Helmet detected / ready |
| Red LED | Helmet missing / blocked |
| Blue LED | Wireless connected |

These indicator meanings are taken from the source PRD. fileciteturn2file0L103-L109

## FR-06 — Buzzer Alert

If the rider attempts ignition without a valid helmet condition, the buzzer shall sound and ignition shall remain blocked. fileciteturn2file0L110-L112

## FR-07 — Auto Disconnect / Restart Prevention

If the rider removes the helmet while the motorcycle is OFF, the system shall prevent a subsequent restart until a valid helmet condition and authentication are restored. fileciteturn2file0L113-L115

## FR-08 — Battery Monitoring

The helmet battery level shall be monitored. A low-battery condition shall produce a warning indication, including LED blinking. fileciteturn2file0L116-L119

# 8. Non-Functional Requirements

| Category | Requirement | Target |
| --- | --- | --- |
| Reliability | Communication success | 99% |
| Performance | Helmet detection | <500 ms |
| Performance | Wireless delay | <300 ms |
| Power | Helmet battery runtime | ≥30 h |
| Security | Communication | Encrypted |
| Security | Identity | Unique device ID |
| Security | Pairing | Pairing protection |
| Safety | Ignition interface | Relay isolated from controller |
| Safety | Electrical protection | Fuse protection |
| Safety | Electrical protection | Reverse-polarity protection |

These targets and safety requirements are from the uploaded PRD. fileciteturn2file0L120-L141

# 9. System Architecture

The product consists of two main units: a battery-powered Helmet Unit and a motorcycle-mounted Bike Unit.

HELMET UNIT
Pressure Sensor + Chin Strap Sensor
            ↓
          ESP32
        ↙   ↓   ↘
 Battery  Status LED  Wireless
                     ↓
               ESP-NOW / BLE
                     ↓
                 BIKE ESP32
              ↙    ↓     ↘
           LEDs  Buzzer  Relay Driver
                         ↓
                  Ignition Switch
                         ↓
                  Motorcycle

The uploaded PRD describes the same helmet-side and motorcycle-side architecture. fileciteturn2file0L142-L162

# 10. Hardware Requirements

| Unit | Required Components |
| --- | --- |
| Helmet | ESP32 |
| Helmet | Pressure sensor |
| Helmet | Li-ion battery |
| Helmet | TP4056 charging module |
| Helmet | LED |
| Helmet | Buzzer (optional) |
| Helmet | Power switch |
| Bike | ESP32 |
| Bike | Relay module |
| Bike | MOSFET driver |
| Bike | Motorcycle battery |
| Bike | Voltage regulator |
| Bike | LEDs |
| Bike | Fuse |

The component list follows the uploaded PRD. fileciteturn2file0L163-L180

# 11. Software Requirements

| Category | Requirement |
| --- | --- |
| Language | Embedded C |
| Primary IDE | Arduino IDE |
| Alternative IDE | PlatformIO / VS Code |
| Libraries | Bluetooth, ESP-NOW, GPIO, EEPROM |
| Firmware responsibilities | Sensor acquisition, authentication, communication, battery monitoring, relay control, indication |

The source PRD lists Embedded C, Arduino IDE, Bluetooth, ESP-NOW, GPIO, EEPROM, PlatformIO and VS Code. fileciteturn2file0L181-L194

# 12. User Flow

Wear Helmet → Sensor Detects Helmet → Helmet ESP32 Sends Authentication → Bike ESP32 Receives Data → Authentication Successful? → YES: Enable Relay → Motorcycle Starts. NO: Relay OFF → Buzzer ON → Engine Cannot Start.

This is the user flow defined in the source PRD. fileciteturn2file0L195-L219

# 13. Use Cases

| Use Case | Flow | Expected Result |
| --- | --- | --- |
| UC-01 Successful Start | Wear helmet → authenticate → ignition enabled | Engine can start |
| UC-02 Helmet Not Worn | Press ignition → helmet absent | Relay OFF + buzzer; bike does not start |
| UC-03 Helmet Battery Dead | No helmet signal → authentication fails | Bike does not start |
| UC-04 Unauthorized Helmet | Unknown helmet attempts connection | Authentication fails; bike remains OFF |

These four use cases are defined in the uploaded PRD. fileciteturn2file0L220-L252

# 14. Constraints

| Constraint | Requirement / Impact |
| --- | --- |
| Battery | Helmet battery requires charging |
| Wireless | Interference may occur |
| Environment | Waterproof enclosure required |
| Temperature | System must account for temperature variations |
| Mechanical | Vibration resistance required |

These constraints are explicitly listed in the source PRD. fileciteturn2file0L253-L258

# 15. Product Risks

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Helmet battery dead | High | Battery indicator / warning |
| Wireless disconnection | Medium | Auto reconnect |
| Sensor failure | High | Sensor diagnostics |
| Relay failure | High | Manual service mode |
| Water damage | High | IP-rated enclosure |

Risk ratings and mitigations are preserved from the source PRD. fileciteturn2file0L259-L271

# 16. Acceptance & Success Criteria

| ID | Criterion | Target |
| --- | --- | --- |
| SC-01 | Ignition authorization | Bike starts only when helmet is worn |
| SC-02 | Helmet detection accuracy | ≥98% |
| SC-03 | Wireless latency | <300 ms |
| SC-04 | Connection establishment | Within 2 s |
| SC-05 | Battery backup | ≥30 h |
| SC-06 | Mechanical robustness | System survives vibration during riding |

These success criteria are taken directly from the uploaded PRD. fileciteturn2file0L272-L280

# 17. Future Enhancements

Potential future upgrades include fingerprint authentication, face recognition, GPS, GSM emergency alerts, alcohol detection, accident detection, crash notification, speed limiting, mobile-app integration, cloud analytics, ride history, theft protection and voice-assistant functionality. These are not part of the current prototype scope. fileciteturn2file0L281-L294

# 18. Estimated Prototype Cost

| Component | Qty. | Approx. Cost (₹) |
| --- | --- | --- |
| ESP32 – Helmet | 1 | 500 |
| ESP32 – Bike | 1 | 500 |
| Pressure/IR Sensor | 1 | 150 |
| Relay Module | 1 | 120 |
| Li-ion Battery | 1 | 350 |
| TP4056 Charger | 1 | 80 |
| LEDs, Buzzer, Wires | — | 200 |
| Voltage Regulator | 1 | 100 |
| Enclosure & Mounting | — | 500 |
| Miscellaneous | — | 500 |
| Total | — | ≈ ₹3,000–₹3,500 |

The cost figures are reproduced from the uploaded PRD and should be treated as an estimate, not a current supplier quotation. fileciteturn2file0L295-L316

# 19. Product Deliverables

| Deliverable | Required |
| --- | --- |
| Hardware prototype | Helmet Unit + Bike Unit |
| Circuit diagram | Yes |
| PCB design | Optional |
| Embedded firmware | Yes |
| System integration | Yes |
| Test report | Yes |
| User manual | Yes |
| Final project report | Yes |
| Presentation | PPT |
| Demonstration | Video |

These deliverables follow the source PRD. fileciteturn2file0L317-L328

# 20. Recommended Technology Baseline

| Area | Baseline |
| --- | --- |
| Helmet sensing | ESP32 + chin-strap magnetic sensor + pressure sensor |
| Communication | ESP-NOW for ESP32-to-ESP32 prototype |
| Bike control | ESP32 + relay/MOSFET driver |
| Security | Unique device IDs + pairing + encrypted communication |

The uploaded PRD recommends this technology stack as a robust, industry-style implementation. fileciteturn2file0L329-L340

# 21. Requirements Traceability Baseline

| Requirement Group | Design Area | Verification Evidence |
| --- | --- | --- |
| FR-01 | Helmet sensing | Detection accuracy/timing test |
| FR-02 | Wireless communication | Range, latency and packet-success test |
| FR-03 | Authentication | Authorized/unauthorized helmet test |
| FR-04 | Ignition control | Relay ON/OFF functional test |
| FR-05/FR-06 | Indicators | LED/buzzer test |
| FR-07 | Restart prevention | Helmet removal/restart test |
| FR-08 | Battery monitoring | Low-battery and runtime test |
| NFR | Performance/security/safety | V&V test report |

# 22. Out of Scope for This PRD

The current product requirements do not define detailed circuit schematics, exact GPIO allocation, packet byte layout, cryptographic key provisioning, PCB layout, firmware API signatures, FMEA, manufacturing procedures or vehicle certification. Those belong in the SDD/DDD and associated engineering documents. This separation keeps the PRD focused on what the product must do rather than exactly how every implementation detail is built.

# 23. Final Product Requirement Statement

The Smart Helmet-Based Motorcycle Ignition System shall provide a reliable embedded safety mechanism that detects helmet use, authenticates the authorized helmet wirelessly, and permits motorcycle ignition only when the required conditions are satisfied. The product shall provide clear rider feedback, monitor helmet battery status, handle communication/sensor faults safely, and meet the defined performance, reliability and environmental targets during verification.

# Document Status

PRD v2.0 — Recommended baseline for proceeding to the Solution Design Document (SDD), followed by Detailed Design, FMEA/Risk Analysis, BOM, Test Plan and Verification & Validation documentation.