SMART HELMET-BASED
MOTORCYCLE IGNITION SYSTEM

Software/Hardware Solution Design Document (SDD)

Version 2.0  |  Engineering Prototype Specification

Author: Yogesh

# Document Control

| Item | Value |
| --- | --- |
| Document | Software/Hardware Solution Design Document |
| System | Smart Helmet-Based Motorcycle Ignition System |
| Version | 2.0 |
| Primary Controller | ESP32 |
| Wireless Options | ESP-NOW / BLE |
| Helmet Power | 18650 Li-ion battery; TP4056 charging; 3.3 V regulation |
| Motorcycle Supply | 12 V battery with buck conversion for controller |
| Target Detection Time | <500 ms |
| Target Wireless Range | 5 m |
| Authentication Timeout | 2 s |
| Proposed Security | AES-128 plus challenge/response for production-oriented prototype |
| Status | Design baseline |

This document refines the uploaded SDD into a clearer implementation-oriented baseline. Existing source architecture and stated parameters are retained; the API contract and message rules below are made more explicit so firmware modules can be implemented and tested consistently.

Source basis: the original SDD defines ESP32, BLE/ESP-NOW, pressure and strap sensing, relay control, packet fields, API concepts, state machine, parameters, error codes and test cases. fileciteturn1file0L12-L25

# 1. Purpose and Scope

The system prevents motorcycle ignition authorization unless an authorized smart helmet is detected as worn and successfully authenticated by the motorcycle controller.

| In Scope | Out of Scope / Future |
| --- | --- |
| Helmet-wear detection | Cloud analytics |
| Helmet-to-bike wireless authentication | Mobile application |
| Battery monitoring | GPS tracking |
| Ignition relay authorization | Accident detection |
| Buzzer/LED status indication | Remote emergency-contact services |
| Fault and timeout handling | Production automotive certification |

The source document's future architecture includes mobile app, cloud, analytics, emergency contact, GPS and accident detection. fileciteturn1file0L404-L422

# 2. System Overview

Two embedded nodes cooperate: a battery-powered Helmet Unit and a motorcycle-powered Bike Unit. The Helmet Unit determines helmet-worn status and transmits an authentication/status message. The Bike Unit validates the message and controls an ignition relay.

The source architecture contains pressure sensor, strap sensor, ESP32, battery and wireless communication on the helmet; the bike side contains ESP32, relay driver, buzzer and LEDs connected to the ignition circuit. fileciteturn1file0L38-L63

# 3. System Architecture

High-level architecture:

Rider → Pressure/Strap Sensors → Helmet ESP32 → Authentication/Communication → Bike ESP32 → Validation → Relay Driver → Ignition Circuit → Motorcycle Engine

Supporting paths: Helmet battery → charging/regulation → Helmet ESP32; Bike 12 V battery → buck converter → Bike ESP32; Bike ESP32 → LEDs/Buzzer.

The original state flow is OFF → READY → AUTHENTICATING → SUCCESS → ENGINE ON, with a failure path back to OFF and alarm indication. fileciteturn1file0L290-L327

## 3.1 Functional Flow

| Step | Condition / Action | Result |
| --- | --- | --- |
| 1 | Power ON | Initialize MCU, sensors and communication |
| 2 | Read sensors | Determine helmet-worn state |
| 3 | Helmet not worn | Remain blocked; indicate E01 |
| 4 | Helmet worn | Prepare authentication message |
| 5 | Transmit | Send status/authentication packet |
| 6 | Bike receives | Validate identity, freshness and integrity |
| 7 | Validation fails | Keep relay OFF; indicate E03/E04 as applicable |
| 8 | Validation succeeds | Authorize ignition |
| 9 | Start request | Relay is enabled for the defined ignition function |
| 10 | Stop/fault | Remove authorization and return to safe blocked state |

## 3.2 Safety Principle

Fail-safe rule: an uncertain or invalid condition must not authorize ignition. Helmet missing, authentication failure, communication timeout, sensor failure, dead/low battery or relay fault must result in ignition authorization being withheld or removed as appropriate. The original SDD includes E01–E06 and tests for wrong helmet, dead battery, sensor failure and communication loss. fileciteturn1file0L377-L403

# 4. Hardware Architecture

| Node | Component | Interface / Role |
| --- | --- | --- |
| Helmet | ESP32 | Main controller |
| Helmet | Pressure sensor | Helmet-worn detection input |
| Helmet | Strap switch/sensor | Secondary wear confirmation |
| Helmet | 18650 Li-ion | Energy source |
| Helmet | TP4056 + regulator | Charging and regulated supply |
| Helmet | Green/Red LEDs | Local status indication |
| Helmet | BLE / ESP-NOW | Wireless communication |
| Bike | ESP32 | Main controller |
| Bike | Relay module/driver | Ignition authorization output |
| Bike | Buzzer | Fault/status alarm |
| Bike | Green/Red LEDs | Status indication |
| Bike | 12 V battery + buck | Bike controller power |

Source pin allocation: helmet pressure sensor GPIO18, strap GPIO19, green LED GPIO2, red LED GPIO4; bike relay GPIO25, buzzer GPIO26, green LED GPIO27, red LED GPIO14. These are the pins stated in the uploaded SDD and should be treated as the current design baseline until hardware validation. fileciteturn1file0L110-L132

# 5. Software Architecture

Firmware is divided into modules so hardware, communication and application logic can be tested independently.

| Module | Responsibilities |
| --- | --- |
| main | Startup, state machine, scheduling |
| sensors | Pressure/strap acquisition, filtering, sensor health |
| battery | Battery estimation and low-battery detection |
| communication | Packet encoding/decoding, transmit/receive, timeout |
| authentication | Identity, freshness, integrity/security validation |
| ignition | Relay authorization and safe disable |
| indication | LED and buzzer patterns |
| diagnostics | Error codes, fault state and event logging |

The source repository structure already separates helmet sensors/communication/battery and bike ignition/authentication/relay modules. fileciteturn1file0L423-L461

## 5.1 State Machine

| State | Entry Condition | Allowed Transition | Output |
| --- | --- | --- | --- |
| OFF | Power-up or stop/fault | Helmet detected → READY | Ignition disabled |
| READY | Helmet detected | Start request → AUTHENTICATING; helmet removed → OFF | Ready indication |
| AUTHENTICATING | Start requested | Valid auth → SUCCESS; timeout/fail → OFF | Ignition disabled |
| SUCCESS | Authentication accepted | Ignition request → ENGINE ON; failure → OFF | Authorization active |
| ENGINE ON | Engine running | Stop/helmet removed/fault → OFF | Defined relay behavior |
| FAULT | Critical sensor/communication/relay fault | Recovery/power cycle | Ignition disabled + alarm |

# 6. Communication Architecture

The source design permits BLE or ESP-NOW. For the prototype, select one transport as the implementation baseline; the application packet contract below remains transport-independent.

Flow: Helmet ESP32 → Authentication/Status Packet → Wireless Link → Bike ESP32 → Decode → Integrity check → Identity/security check → Freshness check → State decision → Relay.

The original SDD specifies Device_ID, Helmet_Status, Battery_Level, Timestamp and CRC, with a 12-byte example packet. fileciteturn1file0L147-L163

## 6.1 Proposed Canonical Packet Contract

To make the contract unambiguous, the following is a refined implementation format. This is a design refinement, not a claim that the original document already implemented every field below.

| Field | Size | Encoding | Requirement |
| --- | --- | --- | --- |
| Version | 1 byte | uint8 | Protocol version; current = 1 |
| MessageType | 1 byte | uint8 | HELLO=0x01, AUTH=0x02, STATUS=0x03, ACK=0x04, ERROR=0x05 |
| DeviceID | 4 bytes | uint32 | Unique authorized helmet identifier |
| HelmetStatus | 1 byte | bit field | bit0: pressure OK; bit1: strap OK; bit2: worn |
| BatteryLevel | 1 byte | 0–100 | Estimated percentage |
| Counter | 4 bytes | uint32 | Monotonic message counter for freshness |
| Timestamp | 4 bytes | uint32 | Optional synchronized/relative time |
| CRC16 | 2 bytes | uint16 | Integrity check over preceding fields |

Total canonical payload: 18 bytes. If AES-based confidentiality/authentication is added, the encrypted/authenticated transport envelope should be specified separately rather than silently changing the payload contract.

## 6.2 Message Semantics

| Message | Direction | Purpose | Bike Response |
| --- | --- | --- | --- |
| HELLO | Helmet → Bike | Announce/establish presence | ACK or challenge |
| AUTH | Helmet → Bike | Prove authorized helmet | Authorization result |
| STATUS | Helmet → Bike | Report worn state/battery/health | ACK optional |
| ACK | Bike → Helmet | Confirm receipt/processing | None |
| ERROR | Bike → Helmet | Report rejected/fault condition | None |

# 7. API Contract

The original SDD defines internal firmware APIs such as initializeSensors(), detectHelmet(), getBatteryLevel(), sendAuthentication(), receivePacket(), verifyHelmet(), enableIgnition(), disableIgnition() and triggerAlarm(). fileciteturn1file0L164-L210 The following contract makes their inputs, outputs and failure behavior explicit.

## 7.1 Helmet Firmware API

| API | Input | Output | Contract |
| --- | --- | --- | --- |
| initializeSensors() | None | bool | true only if required sensors initialize and pass basic self-test |
| detectHelmet() | None | HelmetStatus | Returns pressure_ok, strap_ok, worn, sensor_fault |
| getBatteryLevel() | None | uint8 0–100 | Returns estimated battery percentage |
| sendAuthentication(req) | AuthRequest | Result | Transmits AUTH and returns SENT, TIMEOUT or LINK_ERROR |
| sendStatus() | None | Result | Sends current status/battery/health |
| getDeviceId() | None | uint32 | Returns configured helmet ID |
| enterLowPower() | None | void | Reduces consumption when inactive |

## 7.2 Bike Controller API

| API | Input | Output | Contract |
| --- | --- | --- | --- |
| receivePacket() | None | PacketResult | Returns VALID, INVALID, TIMEOUT or LINK_ERROR |
| verifyHelmet(packet) | Packet | AuthResult | Checks ID, status, freshness, integrity and configured security rules |
| enableIgnition() | None | bool | Enables relay only when authorization state is valid |
| disableIgnition() | Reason | bool | Removes ignition authorization; safe default |
| triggerAlarm(code) | ErrorCode | void | Activates buzzer/indicator pattern |
| getSystemState() | None | State | Returns current state machine state |
| clearFault() | None | bool | Clears recoverable faults only after conditions are healthy |

## 7.3 API Error Contract

| Code | Meaning | Required System Behavior |
| --- | --- | --- |
| E01 | Helmet not worn | Do not authorize ignition |
| E02 | Battery low | Warn rider; do not authorize if configured threshold requires healthy battery |
| E03 | Authentication failed | Relay OFF; alarm/indicator |
| E04 | Connection lost | Authorization expires; relay OFF |
| E05 | Relay failure | Enter FAULT; relay command OFF; require service/recovery |
| E06 | Sensor failure | Enter blocked/fault state; alarm/indicator |

These error identifiers originate in the uploaded SDD. fileciteturn1file0L377-L386

# 8. Authentication and Security Contract

The original document specifies AES-128. fileciteturn1file0L369-L376 For a stronger implementation, use a challenge-response sequence so a captured old packet cannot by itself authorize ignition.

| Phase | Helmet | Bike |
| --- | --- | --- |
| 1 | Send HELLO + DeviceID | Check whether DeviceID is registered |
| 2 | Receive challenge | Generate random challenge |
| 3 | Return AUTH proof over challenge + device secret | Verify proof |
| 4 | Send current worn/battery status | Grant short-lived authorization |
| 5 | Continue periodic STATUS/keepalive | Expire authorization if updates stop |

Security requirement: never treat DeviceID alone as proof of authorization. Protect secrets from being transmitted in plaintext. The exact AES mode/key provisioning method must be finalized in the Detailed Design Document.

# 9. Timing and Performance Requirements

| Parameter | Target |
| --- | --- |
| Helmet detection time | <500 ms |
| Wireless operating range | 5 m target |
| Authentication timeout | 2 s |
| Helmet battery backup | 30 h target |
| Helmet battery | 18650 Li-ion |
| Helmet controller voltage | 3.3 V |
| Bike supply | 12 V |
| Communication | ESP-NOW / BLE |
| Security | AES-128 stated in source; challenge-response recommended |

These targets are taken from the uploaded SDD. They are design targets and must be verified experimentally rather than assumed to be achieved. fileciteturn1file0L358-L376

# 10. Power Architecture

Helmet: 18650 Li-ion → TP4056 charging/protection stage → regulated 3.3 V → ESP32/sensors. Bike: 12 V motorcycle battery → buck converter → ESP32 and control electronics. The original document specifies this topology. fileciteturn1file0L121-L132

Design requirement: verify regulator current capacity, battery protection, charging temperature, standby current and measured runtime before claiming the 30-hour target.

# 11. Fault Handling

| Fault | Detection | Response |
| --- | --- | --- |
| Helmet missing | Pressure/strap state | Block ignition; E01 |
| Low battery | Battery ADC/estimator | E02 + warning; apply configured policy |
| Wrong helmet | Authentication/ID check | E03; relay OFF |
| Wireless loss | Keepalive/timeout | E04; authorization expires |
| Relay failure | Output feedback/current sensing if implemented | E05; FAULT state |
| Sensor failure | Self-test/range/plausibility | E06; relay OFF |

# 12. Verification & Test Plan

| Test ID | Test | Expected Result |
| --- | --- | --- |
| T01 | Helmet worn + authorized | Ignition authorization available |
| T02 | Helmet removed | Ignition remains/returns OFF |
| T03 | Wrong helmet | Ignition blocked; E03 |
| T04 | Helmet battery depleted | Ignition blocked; low-battery/fault behavior |
| T05 | Sensor disconnected/fails | E06; ignition blocked |
| T06 | Wireless link lost | Authorization expires; relay OFF |
| T07 | Authentication timeout | Relay remains OFF |
| T08 | Replay/old counter | Packet rejected |
| T09 | CRC error | Packet rejected |
| T10 | Power-cycle helmet/bike | System returns to safe OFF/initialization state |
| T11 | Range test | Measure maximum reliable range; compare with 5 m target |
| T12 | Detection timing | Measure end-to-end detection and confirm <500 ms target |

The original SDD already specifies tests for helmet worn, helmet removed, wrong helmet, dead battery, sensor failure and Bluetooth loss. fileciteturn1file0L387-L403

# 13. Hardware Safety and Prototype Boundary

The ignition interface is safety-critical. For initial development, validate relay logic using an isolated test load before connecting to a real motorcycle. The prototype should have a clearly defined manual/service bypass only if required for controlled testing, and that bypass must not be presented as a production safety feature.

The relay output in the source design is connected to the ignition wire. fileciteturn1file0L122-L132 Final vehicle wiring, protection, fusing, transient protection and connector selection belong in the Detailed Design Document and must be reviewed before road use.

# 14. Deployment / Repository Structure

Recommended structure:

smart-helmet-system/
  docs/  (SDD, API Contract, Test Plan, FMEA, User Manual)
  firmware/helmet/  (main, sensors, communication, battery)
  firmware/bike/  (main, ignition, authentication, relay)
  hardware/schematics/
  hardware/pcb/
  hardware/bom.xlsx
  diagrams/
  simulations/
  README.md

This preserves the repository organization proposed by the original SDD. fileciteturn1file0L423-L463

# 15. Open Design Decisions

| Decision | Current Baseline | Must Be Finalized Before Prototype Freeze |
| --- | --- | --- |
| Wireless transport | BLE / ESP-NOW | Choose one for primary implementation |
| Helmet sensor type | Pressure + strap | Exact part, mounting and thresholds |
| Battery measurement | Battery percentage | ADC circuit and calibration |
| Authentication | AES-128 stated | Key provisioning + challenge-response details |
| Ignition control | Relay | Exact relay/driver, protection and feedback |
| Low-battery threshold | Not numerically defined | Set and validate threshold |
| Keepalive period | Not numerically defined | Set period and expiry policy |
| Timestamp source | Included in packet | Define clock synchronization/meaning |

# 16. Conclusion

This SDD defines the smart helmet system as two cooperating ESP32 embedded nodes with helmet-wear sensing, wireless authentication, fault handling and controlled ignition authorization. It retains the source document's core architecture and parameters while making the API and communication contract precise enough to serve as the baseline for firmware and hardware detailed design.

Next engineering documents should be: 1) Detailed Design Document, 2) FMEA/Risk & Safety Analysis, 3) BOM and component-selection sheet, 4) Test Plan with measurable acceptance criteria, and 5) User/maintenance manual. The uploaded SDD itself identifies detailed schematics, BOM, communication protocol, firmware architecture, functional-safety concepts, FMEA and V&V as the next level of industry-grade documentation. fileciteturn1file0L464-L488